# Power Calculations

## Battery

2S Li-ion
Nominal = 7.4 V
Max = 8.4 V

Design at 8.4 V.

## DC Motors (4x)

Assumed per motor:
Running current = 0.3 A
Stall current = 1.5 A

Worst case stall:

I_motors = 4 × 1.5 A
I_motors = 6 A

Normal running:

I_motors_run = 4 × 0.3 A
I_motors_run = 1.2 A

## Servo (6 V)

Running ≈ 0.5 A
Stall ≈ 2 A

Worst case:

I_servo = 2 A

## 5 V Rail Load

ATmega ≈ 0.02 A
2 × TB6612 logic ≈ 0.02 A
MPU6050 ≈ 0.005 A
Misc ≈ 0.05 A

Total ≈ 0.1–0.2 A

Use worst case:

I_5V = 0.2 A

## Total Battery Current (Worst Case)

I_total_peak = I_motors + I_servo + I_5V_input

First calculate 5V input current.

## 5 V Buck Regulator

Output:

P_out_5V = V × I
P_out_5V = 5 V × 0.2 A
P_out_5V = 1 W

Assume efficiency η = 80%

P_in_5V = P_out / η
P_in_5V = 1 / 0.8
P_in_5V = 1.25 W

Input current:

I_in_5V = P_in / V_batt
I_in_5V = 1.25 / 8.4
I_in_5V ≈ 0.15 A

## 6 V Buck Regulator (Servo)

Worst case stall:

P_out_6V = 6 V × 2 A
P_out_6V = 12 W

Assume η = 80%

P_in_6V = 12 / 0.8
P_in_6V = 15 W

Input current:

I_in_6V = 15 / 8.4
I_in_6V ≈ 1.79 A

Power dissipated:

P_loss_6V = P_in − P_out
P_loss_6V = 15 − 12
P_loss_6V = 3 W

## Total Battery Peak Current

I_total_peak =
6 A (motors)

- 2 A (servo)

- 0.15 A (5V buck input)

I_total_peak ≈ 8.15 A

## Average Operating Current

Motors running: 1.2 A
Servo running: 0.5 A
5V input: 0.15 A

I_total_avg ≈ 1.85 A

## Trace Width Justification (1 oz copper)

High current paths (VBAT, motor outputs):

Design current ≈ 6–8 A peak
Trace width used = 2 mm

Logic and signal traces:

Current < 0.2 A
Trace width used = 0.25 mm

5V rail:

Current ≈ 0.2 A
Trace width used = 1 mm

## Capacitor Justification

Battery bulk: 470 µF
5V rail bulk: 1000 µF
6V rail bulk: 470 µF

Used to reduce voltage drop during motor and servo current spikes.

Maximum instantaneous current: ≈ 8.15 A
Average operating current: ≈ 1.85 A

Design is within expected limits for 2S 18650 system.